PROBLEM STATEMENT

Day 31 : Always_case

Day 32 : Always_case2

Day 33 : Always_casez

Day 34 : Always_nolatches

Day 35 : Conditional

Day 36 : Reduction

Day 37: gates100

Day 38 : vector100r

Day 39 : popcount255

Day 40: Adder100i

DAY:31 Always_case

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [2:0] sel,

input [3:0] data0,

input [3:0] data1,

input [3:0] data2,

input [3:0] data3,

input [3:0] data4,

input [3:0] data5,

output reg [3:0] out

);

always@(*) begin

case(sel)

0 : out = data0;

1 : out = data1;

2 : out = data2;

3 : out = data3;

4 : out = data4;

5 : out = data5;

default : out = 0;

endcase

end

endmodule

Timing Diagram:

DAY:32 Always_case2

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [3:0] in,

output reg [1:0] pos

);

always @(*) begin

case (in)

4'h0: pos = 2'h0;

4'h1: pos = 2'h0;

4'h2: pos = 2'h1;

4'h3: pos = 2'h0;

4'h4: pos = 2'h2;

4'h5: pos = 2'h0;

4'h6: pos = 2'h1;

4'h7: pos = 2'h0;

4'h8: pos = 2'h3;

4'h9: pos = 2'h0;

4'ha: pos = 2'h1;

4'hb: pos = 2'h0;

4'hc: pos = 2'h2;

4'hd: pos = 2'h0;

4'he: pos = 2'h1;

4'hf: pos = 2'h0;

default: pos = 2'b0;

endcase

end

endmodule

Timing Diagram:

DAY:33 Always_casez

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [7:0] in,

output reg [2:0] pos

);

always @(*) begin

casez(in)

8'bzzzzzzz1 : pos = 0;

8'bzzzzzz1z : pos = 1;

8'bzzzzz1zz : pos = 2;

8'bzzzz1zzz : pos = 3;

8'bzzz1zzzz : pos = 4;

8'bzz1zzzzz : pos = 5;

8'bz1zzzzzz : pos = 6;

8'b1zzzzzzz : pos = 7;

default : pos = 0;

endcase

end

endmodule

Timing Diagram:

DAY:34 Always_nolatches

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [15:0] scancode,

output reg left,

output reg down,

output reg right,

output reg up   );

always @(*) begin

left = 0; down = 0; right = 0; up = 0;

case(scancode)

16'he06b : left = 1;

16'he072 : down = 1;

16'he074 : right = 1;

16'he075 : up = 1;

default : ;

endcase

end

endmodule

Timing Diagram:

DAY:35 Conditional

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [7:0] a, b, c, d,

output [7:0] min );

wire [7:0] w1, w2, w3;

assign w1 = (a < b) ? a : b;

assign w2 = (c < w1) ? c : w1;

assign w3 = (d < w2) ? d : w2;

assign min = w3;

endmodule

Timing Diagram:

DAY:36 Reduction

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module (

input [7:0] in,

output parity );

assign parity = ^in;

endmodule

DAY:37 Gates100

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module(

input [99:0] in,

output out_and,

output out_or,

output out_xor

);

assign out_and = &in;

assign out_or  = |in;

assign out_xor = ^in;

endmodule

Timing Diagram:

DAY:38 vector100r

Problem Statement:

Given a 100-bit input vector [99:0], reverse its bit ordering.

Module Declaration:

Solution:

Code:

module top_module (

input [99:0] in,

output reg [99:0] out

);

always @(*) begin

for (int i=0 ; i < $bits(out) ; i++)

out[i] = in[$bits(out)-i-1];

end

endmodule

DAY:39  popcount255

Problem Statement:

A "population count" circuit counts the number of '1's in an input vector. Build a population count circuit for a 255-bit input vector.

Module Declaration:

Solution:

Code:

module top_module(

input [254:0] in,

output [7:0] out

);

always @(*) begin

out = 0;

for(int i = 0; i < $bits(in); i++) begin

if(in[i])

out = out + 8'b1;

else

out = out + 8'b0;

end

end

endmodule

Timing Diagram:

DAY:40 Adder100i

Problem Statement:

Module Declaration:

Solution:

Code:

module top_module(

input [99:0] a, b,

input cin,

output [99:0] cout,

output [99:0] sum

);

genvar i;

assign sum[0] = a[0]^b[0]^cin;

assign cout[0]=a[0]&b[0] | a[0]&cin | b[0]&cin;

generate

for(i=1; i<100 ; i=i+1)begin:FA

assign sum[i] = a[i]^b[i]^cout[i-1];

assign cout[i] = a[i]&b[i] | a[i]&cout[i-1] | b[i]&cout[i-1];

end

endgenerate

endmodule